FIBER OPTICS

Principle of Optical Fiber

Optical fiber is a cylinder of transparent dielectric medium and designed to guide visible and infrared light over long distances. Optical fibers work on the principle of total internal reflection.

Optical fiber is very thin and flexible medium having a cylindrical shape consisting of three sections

- The core material

- The cladding material

- The outer jacket

Critical angle (θc):

The angle of incidence for an optical ray travelling from denser medium to rarer medium  is called the critical angle (θc) when the angle of refraction is 900

If the angle of incidence is more than the critical angle then the optical ray is reflected back into the same medium. This is called total internal reflection. Let the reflective indices of core and cladding materials be n1 and n2 respectively.

According to law of refraction,

n1 Sinθ1= n2 Sinθ2

Here θ1= θc     θ2=90

n1 Sinθ1= n2 Sin(90)

Sinθc= n2n1

θc =Sin-(n2n1)        --------- (1)

Equation (1) is the expression for condition for total internal reflection.

Acceptance Angle and Acceptance Cone:

Acceptance angle: Acceptance angle is the maximum angle of launch of light beam at its end of the fiber and to enable the entire light to propagate through the core. Fig. shows longitudinal cross section of the launch of a fiber with a ray entering it. The light is entered from a medium of refractive index n0 (for air n0=1) into the core of refractive index n1. The ray (OA) enters with an angle of incidence to the fiber end face i.e. the incident ray makes angle  with the fiber axis which is nothing but the normal to the end face of the core. Let a right ray OA enters the fiber at an angle to the axis of the fiber. The end at which light enter the fiber is called the launching pad.

Let the refractive index of the core be n1 and the refractive index of cladding be n2. Here n1>n2. The light ray reflects at an angle   and strikes the core cladding interface at angle θ. If the angle θ is greater than its critical angle θc, the light ray undergoes total internal reflection at the interface.

According to Snell’s law

n0Sinαi= n1Sinαr                                                                      → (2)

From the right angled triangle ABC

αr+ θ= 900

αr= 90-θ                                                           → (3)

Substituting (3) in (2), we get

n0Sinαi= n1Sin(90-θ)

= n1Cosθ

Sinαi= n1n0  Cosθ                                 →(4)

When θ=θc then  αi=αmax

Sinαmax= n1n0  Cosθc                                                           →(5)

From equation (1) Sinθc = n2n1

Cosθc= 1-Sin2θc =1-(n2n1)2  =n12-n22n1         →(6)

Substitute equation (6) in equation (5)

Sinαmax =(n1n0) n12-n22n1   = n12-n22n0                      →(7)

If the medium surrounding fiber is air, then n0= 1

Sinαmax = n12-n22                                           →(8)

This maximum angle is called the acceptance angle or the acceptance cone half angle of the fiber.

The acceptance angle may be defined as the maximum angle that a light ray can have with the axis of the fiber and propagate through the fiber. Rotating the acceptance angle about the fiber axis (fig .8.13) describes the acceptance cone of the fiber. Light launched at the fiber end within this acceptance cone alone will be accepted and propagated to the other end of the fiber by total internal reflection. Larger acceptance angles make launching easier. Light gathering capacity of the fiber is expressed in terms of maximum acceptance angle and is termed as “Numerical Aperture”.

Numerical Aperture

Numerical Aperture of a fiber is measure of its light gathering power. The numerical aperture (NA) is defined as the Sin of the maximum acceptance angle.

Numerical aperture (NA) =  Sinαmax = n12-n22   			→(9)

= (n1- n2) (n1+ n2)

=((n1+ n2) n1∆)                        → (10)

Where   ∆=  (n1-n2)n1 called as fractional differences in refractive indices n 1and n2are the refractive indices of core and cladding material respectively.

As  n 1≈ n2, we can take n1+ n2= 2n1

Then numerical aperture = 2n12∆ = n12∆                                                  → (11)

Numerical aperture is a measure of amount of light that can be accepted by a fiber. From equation (9) it is seen that numerical aperture depends only on the refractive indices of core and cladding materials and it is independent on the fiber dimensions. Its value ranges from 0.1 to 0.5. A large NA means that the fiber will accept large amount of light from the source.

Step index fibers and Graded index fiber -transmission of signals in them:

Based on the variation of refractive index of core, optical fibers are divided into 2 types

They are (1) Step index fibers                (2) Graded index fibers

Again, based on the mode of propagation, all these fibers are divided into two types.

They are (1) single mode                        (2) multimode fibers.

Differences between step index fiber and graded index fibers:

| Step index fiber | Graded index fiber |
| --- | --- |
| 1.The refractive index of core is uniform and step or abrupt change in refractive index takes place at the interface of core and cladding in step index fibers. | 1. The refractive index of core is non uniform, the refractive index of core decreases radially from the axis of the fiber to its surface. |
| 2. The light rays propagate in zigzag manner inside the core. The rays travel in the fiber as meridional rays they cross the fiber axis for every reflection. | 2. Light rays propagate in the form of skew rays or helical rays. They will not cross the fiber axis |
| 3. The core diameter of this fiber is about 2 to 8µm and outer diameter of cladding is 60 to 70 µm. | 3. The diameter of core varies from 50 to 200µm and outer diameter of cladding varies from 100 to 250 µm. |

Difference between single mode fibers and multi-mode fibers:

| Single mode fiber | Multimode fiber |
| --- | --- |
| - In single mode fiber there is only one path for ray propagation | - In multimode fiber, large number of paths is available for light ray propagation. |
| - A single mode step index fiber has less core diameter (< 10 µm) and the difference between the reflective indices of core and cladding is very small. | - Multi mode fibers, large number of paths are available for light ray propagation. |
| - In single mode fibers, there is no dispersion. | - There is signal distortion and dispersion takes place in multimode fibers. |
| - The band width is about 50 MHz for multimode step index fiber where as it is more than 1000 MHz km in case of single mode step index fiber. | - The band width of the fiber lies in between 200 MHz km to 600 MHz km even though theoretically it has an infinite bandwidth. |
| - NA of multimode step index fiber is more where as in single mode step index fibers, it is very less. | - NA of graded index fibers is less. |
| - Launching of light into single mode fibers is difficult. | - Launching of light into multimode fibers is easy. |
| - Fabrication cost is very high. | - Fabrication cost is less |

Energy losses in Optical fibers:

1. Distortion losses:-

When a pulse is launched at one end of the fiber and collected at the other end of the fiber, the shape and size of the pulse should not be changed. Distortion of signals in optical fiber is an undesired feature. If the output pulse is not same as the input pulse, then the pulse is distorted. If the refractive index of the core is not uniform most of the rays will travel through the medium of lower refractive index region. Due to this the rays which are travel in fiber will become broadened. Because of this the output pulses will no longer matches with the input pulses. The distortion takes place due to the presence of imperfections, impurities and doping concentrations in fiber crystals. Distortion is large in multi mode than in single mode fiber.

2. Transmission losses (attenuation):-

The attenuation or transmission losses may be classified into two categories

i ) Absorption losses

ii) Scattering losses

i) Absorption losses:-

Absorption is a characteristic possessed by all materials every material in universe absorb suitable wavelengths as they incident or passed through the material. Three different mechanisms contribute to absorption losses in glass fiber. These are Ultraviolet absorption, IR absorption and Ion resonance absorption.  In the same way core material of the fiber absorbs wavelengths as the optical pulses or wavelengths pass through it.

ii) Scattering losses:-

The core medium of the fiber is made of glass or silica. In the passage of optical signals in the core medium if crystal defects are encountered, they deviate from the path and total internal reflection is discontinued, hence such signals will be destroyed by entering into the cladding however attenuation is minimum in optical fibers compared to other cables.

Dopants added to silica to modify the refractive index also cause fluctuation in refractive index. These inhomogenities act as reflecting and refracting facets to scatter a small portion of the light passing through the glass, contributing for the scattering losses.

ii) Bending losses:-

The distortion of the fiber from the ideal strait line configuration may also result in fiber. Let us consider a wavefront that travels perpendicular to the direction of propagation. In order to maintain this, the part of the mode which is on the outside of the bend has to travel faster than that on the inside. As per the theory each mode extends an infinite distance into the cladding though the intensity falls exponentially. Since the refractive index of cladding is less than that of the core (n1>n2), the part of the mode traveling in the cladding will attempt to travel faster. As per Einstein’s theory of relativity since the part of the mode cannot travel faster the energy associated with this particular part of the mode is lost by radiation.

Attenuation loss is generally measured in terms of decibels (dB), which is a logarithmic unit.

Loss of optical power = -10 log (Pout/ Pin ) dB

Where  Pout is the power emerging out of the fiber.       Pin is the power launched into the fiber.

Application of Fiber optic sensors and detector:

Sensors are devices used to measure or monitor quantities such as displacement pressure, temperature, flow rate liquid level, chemical composition etc. A smoke detector and pollution detector can be made from fibers.   There are two types of fiber optics sensors.

- Intrinsic or Active sensors:

Here guided like in the fiber gauge modulated by the variable to be measured and then de modulated. Thus sensing and simultaneous measurements of taking place in the active sensors. They are

- Intensity modulate sensors:

Which are based on the change in absorption or transmission of light, refractive index, temperature etc.

- Phase modulated sensors:

Which involve the interference between signal and reference in the interferometer leading to a shift in the interference fringes. These will have high sensitivity.

- Polarization modulated sensors:

Which involve the change in polarization state of the guided signal by the variable.

3. Optical Communications: In recent years optical fibers are being used instead of copper cables, where light signals replace electrical signals.

4. Medical Field: The fiber optic technology is  used in medical diagnostics and medical procedures. It is used in Endoscopy, Cardiology, Ophthalmology  and treatment of Cancer.

5. Military Field: Fiber guided missiles are used in recent wars.

#### 6. Lighting and Decorations:

The use of fiber optics in the area of decorative illumination has also grown over the years. Fiber optic cables provide an easy, economical and attractive solution to lighting projects. As a result, they are widely used in lighting decorations and illuminating Christmas trees.

#### 7. Mechanical Inspections:

Fiber optic cables are widely used in the inspection of hard-to-reach places. Some such applications are on-site inspections for engineers and also inspection of pipes for plumbers.

#### 8. Automotive Industry:

Fiber optic cables play an important role in the lighting and safety features of present day automobiles. They are widely used in lighting, both in the interior and exterior of vehicles.

Fiber optic communication:

Optical fibers are very attractive alternatives to twisted wire or coaxial cables in communication links. In optical fibers were developed with losses less than 1dbkm-1 and with high information carrying capacity (bandwidths of the order of 1014Hz). A schematic diagram of an optical transmission system is shown in fig.

Optical communication system basically consists of three main parts.

- Transmitter 2. Optical fiber and 3. Receiver.

Transmitter: Transmitter consist of an input signal, optical source usually an LED or semiconductor laser and modulator. Modulator modulates the input signals in the form of light signals.

Optical fiber: Light signals obtained are transmitted through optical fiber.

Receiver: At the receiving end, the optical receiver has three functions, namely (a) the conversion of optical signal into electrical signal with the help of photodetectors, (b) amplification and (c) demodulation of the signal received to retrieve the original signal.

Sensors for Structural Health Monitoring (SHM) using optical fibers:

Optical fiber sensors (OFS) have become one of the most powerful tools in modern Structural Health Monitoring due to their high sensitivity, immunity to electromagnetic interference, ability to survive harsh environments, and capacity for long-distance, distributed measurements.

Structural Health Monitoring involves the continuous assessment of structures—such as bridges, buildings, dams, aircraft, wind turbines, and pipelines—by measuring parameters like strain, vibration, temperature, cracks, corrosion, and deformation.